Converting from SageDCT to Primeencoder the easy way.

SageDCT and Primeencoder are both network encoders. I would like to thank the programmers for these 2 great tools that have mannaged to keep Sagetv alive.

Converting.

There is no need to make any changes to the sage.properties file if you are already running SageDCT.

1. Shutdown and disable SageDCT service

2. Look for thes lines in the Sage properties files take note of the ports and HD Prime ID... Below are mine yours will be different.

mmc/encoders/123456970/encoding_host=192.168.0.100\:6970

mmc/encoders/123456970/video_capture_device_name=SageDCT-HDHomeRun Prime Tuner 13147C7B-1

mmc/encoders/123456972/encoding_host=192.168.0.100\:6972

mmc/encoders/123456972/video_capture_device_name=SageDCT-HDHomeRun Prime Tuner 13147C7B-0

mmc/encoders/123456975/encoding_host=192.168.0.100\:6975

mmc/encoders/123456975/video_capture_device_name=SageDCT-HDHomeRun Prime Tuner 13147C7B-2

3. Now open in notepad the PrimeNetEncoder.properties ...

make sure tuners are enabled if need be enable them.

tuner0.enabled=True

tuner1.enabled=True

tuner2.enabled=True

Make port changes and tuner Id changes to your

PrimeNetEncoder.properties.. You can then setup PrimeNetEncoder  to run as a runnable task or batch job I recommend batch job at startup.

#

#Example Setings file for PrimeNetEncoder

#

tuners.count=3

tuner0.name=HDHomeRun Prime Tuner 0

tuner0.port=6972

tuner0.enabled=True

tuner0.id=13147C7B

tuner0.number=0

tuner0.transcoder.port=5000

tuner0.transcode.enabled=False

tuner0.transcode.deinterlace=True

tuner0.transcode.bitrate=4000

# Example for h.264 safe scaling to 720p: trunc(oh*a/2)*2:720

# A setting of blank will do no scaling

tuner0.transcode.scaling=

tuner0.transcode.preset=ultrafast

# Codec is an experimental setting for testing hardware encoding

# There are development builds of ffmpeg that incude those features

tuner0.transcode.codec=libx264

tuner1.name=HDHomeRun Prime Tuner 1

tuner1.port=6970

tuner1.id=13147C7B

tuner1.number=1

tuner1.enabled=True

tuner1.transcoder.port=5010

tuner1.transcode.enabled=False

tuner1.transcode.deinterlace=True

tuner1.transcode.bitrate=4000

tuner1.transcode.scaling=

tuner1.transcode.preset=ultrafast

tuner1.transcode.codec=libx264

tuner2.enabled=True

tuner2.id=13147C7B

tuner2.number=2

tuner2.name=HDHomeRun Prime Tuner 2

tuner2.port=6975

tuner2.transcoder.port=5020

tuner2.transcode.enabled=False

tuner2.transcode.deinterlace=True

tuner2.transcode.bitrate=4000

tuner2.transcode.scaling=

tuner2.transcode.preset=ultrafast

tuner2.transcode.codec=libx264

HDHomeRunConfig.path=C\:\\Program Files\\Silicondust\\HDHomeRun\\hdhomerun_config.exe

ffmpeg.path=ffmpeg.exe

# This setting is in miliseconds.

# Increasing setting may result in less halts in starting recording

ffmpeg.delay=1000

discovery.port=8271