1.	From the main screen, select File then Preferences

2.	Select the IR Output tab. If USB-UIRT is not in the box, click Change Output Device and select it

from the list. Click Edit IR Codes

3.	I have two Remotes currently setup. Start by Click Add Remote

4. 	Give the new remote a Short and Long Name

5.	Select the new remote and click Add IR

6.	Select Learn IR directly from a Remote Control

7.	At this point, you will teach Hip the IR signals from your remote so you need to have

the remote of the pointing directly at USB-UIRT. Press and hold the key you want

Hip to learn.

8.	Continue holding until you are told it is complete

9.	Once you are finished you will see the code added to the new remote. Repeat steps 6 - 9

until all keys are learned. It is a good idea to point the USB-UIRT blaster at the box you want to

control and use the Test IR button to confirm it was properly learned.

10.	Go back to Preference and click Alternate Input tab. Make sure you have MCE select in the

box. Click Add Value. A window will appear asking for you to input a value.

11.	Launch HipWSend. Type a number you want to link to the learned IR code. I used the

number nine. Click Send to Hip. The previous window asking for input should now ask you to confirm

your input (assuming you have Confirm Read checked in the Alternate Input window). If so send

it again from HipWSend.

12. 	In this window 31734.1 is what Hip learned from HipWSend. Click the button

next to the Primary Command

13.	Click Send IR

14.	Select the Remote that has the IR code you want to send and then select the specific

IR you want to send Click Select IR.

15.	Click OK

16.	Give the command a Description and click OK.

17.	Now the command is mapped to the corresponding IR code. Repeat steps 11-17 for the rest

of the keys. Once you have mapped the keys you can now point the USB-UIRT Blaster to the

box you want to control and use HipWSend to confirm that the numbers work.